SEMESTER 1 MODULE 4

Cable Testing

v. 3.1.1

| 4.1.1 | Define period as it relates to the measurement of ocean waves. | The period of the waves is the amount of time between each wave, measured in seconds. |
| --- | --- | --- |
|  | Define frequency. | The frequency is the number of waves that reach the shore each second, measured in hertz (Hz). |
|  | Define Hertz (Hz). | 1 Hz is equal to 1 wave per second, or 1 cycle per second. |
|  | The amplitude of an electrical signal still represents height, but it is measured in what? | in volts (V) |
|  | The period is the amount of time that it takes to complete 1 cycle. This is measured in what? | seconds. |
|  | The frequency of an electrical signal is ________ ______________________ This is measured in Hz. | the number of complete cycles per second. |
|  | If a disturbance is deliberately caused, and involves a fixed, predictable duration, it is called a ________. | pulse |
| 4.1.2 | List four descriptors of Sine waves. | - periodic, which means that they repeat the same pattern at regular intervals. - vary continuously, which means that no adjacent points on the graph have the same value. - graphical representations of many natural occurrences that change regularly over time. - analog waves |
|  | Draw an example of a Sine wave. |  |
|  | List four descriptors of Sine waves. | - Periodic - Do not continuously vary with time. - The wave maintains one value and then suddenly changes to a different value. - After a short amount of time it changes back to the original value. |
|  | Square waves represent what ? | digital signals, or pulses. |
|  | Like all waves, square waves can be described in terms of amplitude, period, and frequency. |  |
|  | Draw an example of a Square wave. |  |
| 4.1.3 | Base 2 is what type of numbering system? | Binary |
|  | Base 10 is what type of numbering system? | Decimal |
|  | Base 16 is what type of numbering system? | Hexadecimal |
| 4.1.5 | What is an important electronic device used to view electrical signals such as voltage waves and pulses. | An oscilloscope |
| 4.1.7 | Noise related to communications refers to what? | undesirable signals. |
|  | List the possible sources of noise: | - Nearby cables that carry data signals - RFI from other signals that are transmitted nearby - EMI from nearby sources such as motors and lights - Laser noise at the transmitter or receiver of an optical signal |
|  | Define white noise. | Noise that affects all transmission frequencies equally |
| 4..1.8 | Analog bandwidth could be used to describe the range of frequencies transmitted by what? | by a radio station or an electronic amplifier. |
|  | What measures how much information can flow from one place to another in a given amount of time? | Digital bandwidth |
| 4.2.1 | Data signals are represented by voltage levels that represent what? | binary ones and zeros. |
|  | New cable installations should be made with the best _____________, ______________ and ____________. | cable, connectors, and interconnect devices |
|  | List the advantages of Coaxial cable. | - less noisy - reduced signal loss |
|  | List the disadvantages of Coaxial cable. | - more expensive - The need to ground the shielding and the bulky size of coaxial cable make it more difficult to install than other copper cabling. |
|  | List the two types of twisted-pair cable. | shielded twisted-pair (STP) and unshielded twisted pair (UTP). |
|  | Fiber-optic cable increases and decreases the intensity of _________ to represent binary ones and zeros in data transmissions. | of light |
|  | Pptical fiber is often used between __________ and __________________ within a building. As costs decrease and speeds increase, optical fiber may become a more commonly used LAN media. | buildings and between floors |
| 4.2.2 | What is the decrease in signal amplitude over the length of a link. | Attenuation |
|  | What contributes to attenuation? | - Long cable lengths - High signal frequencies - Signal energy loss - Impedence caused by defective connectors |
|  | Define Impedance. | Impedance is a measurement of the resistance of the cable to alternating current (AC) and is measured in ohms. |
|  | If a connector is improperly installed on Category 5, it will have a different impedance value than the cable. This is called what? | an impedance discontinuity or an impedance mismatch. |
| 4.2.3 | The transmission of signals from one wire to a nearby wire is called what? | Crosstalk |
|  | Crosstalk is more destructive at _________ transmission frequencies. | higher |
|  | Twisted wire pairs in a cable are also more resistant to what? | crosstalk or noise signals from adjacent wire pairs. |
|  | When connectors are attached to the ends of UTP cable, the wire pairs should be what? | untwisted as little as possible to ensure reliable LAN communications. |
| 4.2.4 | List the three types of crosstalk. | - Near-end Crosstalk (NEXT) - Far-end Crosstalk (FEXT) - Power Sum Near-end Crosstalk (PSNEXT) |
| 4.2.5 | List the ten primary test parameters that must be verified for a cable link to meet TIA/EIA standards. | - Wire map - Insertion loss - Near-end crosstalk (NEXT) - Power sum near-end crosstalk (PSNEXT) - Equal-level far-end crosstalk (ELFEXT) - Power sum equal-level far-end crosstalk (PSELFEXT) - Return loss - Propagation delay - Cable length - Delay skew |
|  | A NIC transmits signals on ___________. | pins 1 and 2 |
|  | A NIC receives signals on ____________. | pins 3 and 6 |
| 4.2.7 | Testers measure the length of the wire based on the _______________ as measured by a Time Domain Reflectometry (TDR) test, not by the physical length of the cable jacket. | electrical delay |
|  | The TDR test is used not only to determine length, but also to identify what? | the distance to wiring faults such as shorts and opens |
| 4.2.8 | What are the main cause of light reflection and signal strength loss in optical fiber and with UTP cable? | improperly installed connectors |
| 4.2.9 | Category 6 cable must pass the tests with higher scores to be certified. Category 6 cable must be capable of carrying frequencies up to ___________ and must have ____________________________. | 250 MHz lower levels of crosstalk and return loss |