INTERNETWORKING I

CHAPTER 5 STUDY GUIDE

Name four types of media used in LANS/WANS today:

STP, UTP, coaxial, fiber, wireless

What is the maximum cable run for UTP? In meters: 100 In feet: 382

How is coaxial cable constructed?

A hollow outer cylinder conductor that surrounds a single inner wire made of two conducting elements, one of which is copper conductor. Around the core conductor, it has a layer of flexible insulation, then a layer of woven copper braid or metallic foil (which acts as a second conductor), and then finally the jacket.

What is one advantage of coaxial over UTP?

It can be run further without repeaters (185 meters); its technology is well known.

What is one disadvantage of coaxial over UTP?

It is more expensive; it is harder to work with; if the connector is not installed correctly, it allows more noise on the wire than UTP does.

What is one BIG advantage of fiber media over copper media?

It is not susceptible to EMI or RFI; it is capable of higher data rates.

Why does TIA/EIA recommend using fiber instead of copper media for backbone?

Because it is not susceptible to EMI or RFI and the runs can be MUCH longer than for either twisted pair or coax.

What are the five categories of media specified by 568-A? Which one(s) is (are) recommended by Cisco for use in Ethernet LANs?

- Cat 1 (voice only)

- Cat 2

- Cat 3 (recognized for LAN [data] usage)

- Cat 4 (STP for LANs)

- Cat 5 (recognized for LAN [data] usage; most frequently recommended and implemented in Ethernet LANs today)

According to 568-A, what is the maximum distance for horizontal cabling runs for UTP (in meters)?

90 meters

What kind of terminator is used with 10BaseT?

Registered Jack (RJ)-45

What kind of terminator is used with coaxial cable?

BNC (British Naval Connector)

Why are the wire pairs in UTP twisted?

For shielding (cancellation effect) to reduce noise problems which result in signal degradation

What is the definition of a patch panel? At what layer of the OSI model do patch panels fall?

A convenient grouping of RJ-45 jacks which are usually rack-mounted. They are at Layer 1 (physical).

What are located on the backs of patch panels? What is their purpose?

Punch-down blocks. They are where you wire in cables usually running from devices.

What are repeaters/hubs used for in a LAN?

To extend the size of the LAN so it can extend further.

Where would you normally find a hub in a LAN that uses a star topology?

In the center.

What happens to the data when a collision occurs?

It’s destroyed bit-by-bit.

What is a collision domain?

The area within a network where a data packet originates and can collide with another packet. It includes all shared media.

What device(s) can you add to a LAN to increase the number of collision domains (which will cut down on the size of collision domains)?

Intelligent networking devices such as routers and intelligent switches.

List seven types of topologies used in networking:

- bus

- ring

- dual-ring

- star

- extended star

- mesh

- cellular

What is the definition of a bus topology?

All the nodes are directly connected to one physical link.

What is the definition of a ring topology?

A single closed ring consisting of nodes and links with each node connected to only two adjacent nodes.

What is the definition of a dual-ring topology?

Two concentric rights, each of which is linked only to its adjacent ring neighbor. The two rings are not connected.

What is the definition of a star topology?

Has a central node with all links radiating from it and allows no other links.

What is the definition of an extended star topology?

Repeats a star topology except that each node that links to the center of the extended star is also the center of its own star.

What is the definition of a mesh topology?

Every node is directly liked to every other node on the network.

What is one disadvantage of using a star topology?

It takes lots of media; if the central node fails, the whole network is brought down.

What is the wiring scheme for a straight through cable? (Use colors or numbers or both)

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| White orange | orange | white green | blue | white blue | green | white brown | Brown |

What is the wiring scheme for a crossover cable? (Use colors or numbers or both)

| 3 | 6 | 1 | 4 | 5 | 2 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| White green | Green | White orange | blue | white blue | Orange | white brown | Brown |

What is the wiring scheme for a rollover cable? (Use colors or numbers or both)

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Orange | White orange | white green | blue | white blue | green | white brown | Brown |